Assignment 1

Analyse the operation of the enclosed combinational logic. Present for F:

a) logic function as minimised sum of products

b) truth table

c) Karnaugh map

d) minimised logic diagram using 2- and 3-input NAND-gates

e) minimised logic diagram using 2- and 3-input NOR-gates

Assignment 2

The enclosed symbol represents a synchronous 4-bit counter which has two control inputs in addition to the clock signal. The notation 1,2+2CT=0 next to the clock input means that when the control input G1 is a logic one and the control input G2 is logic zero the binary output representing the content CT(1,2,4,8) of the counter is incremented by one with the next rising edge of the clock signal. If the control input G2 is a logic one the content of the counter is zeroed with the next rising edge of the clock signal.

a) Using the enclosed symbol design a counter CTRDIVx, which carries out the counting sequence 0,1,2,3,4,5,6,7,8,9,0,1,2,3... In addition to the counter you can use one additional logic gate.

b) what is x in the definition CTRDIVx of a)?

c) Also design a synchronous counter system CTRDIVy, which can be used for counting the number of seconds within a minute or the number of minutes within an hour, for example, that is enhance the counter logic of a) with another similar counter. The counting sequence of the two counters must be:

00,01...09,10,11...19,20,21,29...30,31...39,40,41...49,50,51...59,00...

In addition to the second counter you can use one more logic gate.

d) what is the value of y in the definition CTRDIVy of c)?

e) how many flip-flops is really needed at least to realise the counter system of b)? Explain!

Assignment 3

- Convert a binary number 101011010   to a decimal system number.

- Convert a decimal number 535  to a binary number.

- The display of a measuring equipment is capable to show numbers in the range         -1.9999 … +1.9999. How many bits are needed to represent them?

- What is the arithmetic sum and product of binary numbers 110100 and 111101?

- How many Full-Adders are needed to implement the adder which is capable to perform the summing operation described in point d of this assignment?

Assignment 4.

- If a digital clock displays time 16:19:23, what is the content of binary vectors at points a, b, c, d, e, f, g, h, i, j, k, l, m, and n in the RTL diagram below. Give the values of binary vectors in binary and also in hexadecimal form.

- If the frequency f of the crystal oscillator is 1 MHz, what has to be the value of N of the frequency divider CTRDIVN? How many bits are needed to implement the divider?

- How many D Flip-Flops is needed to implement a digital clock? Note, that the divider CTRDIVN also needs DFFs to be implemented.